PERFORMANCE SERIES

XIRCOM CREDITCARD TOKEN RING ADAPTER IIps

VERSION 2.19

Additional Information

SmartRing Software version 2.0

November 8, 1994

========================================================================

Thank you for purchasing the Xircom CreditCard Token Ring Adapter IIps.

Xircom has done everything to ensure that our CreditCard Token Ring

Adapter is compatible with notebooks that support PCMCIA Release 2.0

to 2.1 I/O slots.  Xircom is committed to making all of its networking

products compatible with the widest range of mobile PCs possible.

As a company, we are equally committed to customer satisfaction and

timely response to the needs and concerns of our customers.

As this technology evolves and more PCMCIA Release 2.0/2.1 PCs become

available, you can be assured Xircom will continue to provide the

highest level of hardware and software compatibility in the industry.

If you have compatibility issues related to your specific environment,

or you have general questions, please contact Xircom Technical Support.

Information on Xircom Technical Support is located at the end of this

file.

Please refer to your network operating system manual and your Xircom

User's Guide for instructions on installation of the drivers contained

on this diskette.  This .DOC file contains information that may not

be in your User's Guide.

Remember to register your Xircom CreditCard Token Ring Adapter

to receive information about future software upgrades and

new products.

CONTENTS:

---------

1)  CreditCard Token Ring Adapter Characteristics

2)  Features of this Release

3)  Microsoft Notes

4)  OS/2 Notes

5)  AS/400 Notes

6)  CTTEST.EXE & CTSETUP.EXE Configuration

7)  Troubleshooting

8)  Machine Specific Notes

9)  Xircom Technical Support

1)   CREDITCARD TOKEN RING ADAPTER CHARACTERISTICS

=============================================

This release of Xircom's CreditCard Token Ring Adapter has the

following characteristics:

- Card and Socket Services set versions 2.0 to 2.1 compatibility.

Card and Socket Services is a software specification defined by the

PCMCIA organization.  Its purpose is to serve as an intermediary

between the network device driver and the system resources required

by the PCMCIA Adapter.

- Intended for use with Intel 82365 PCIC, Sharp PHIC, Databook TCIC,

Cirrus Logic CL-PD6710 or CL-PD6720 or Toshiba ICCNT PCMCIA

interface chips.

- Variable interrupts, 3 to 15, default interrupt 5.

- Variable I/O window, 32 bytes needed, default base at 300h.

- Requires an 8Kbyte memory window on an 8Kbyte boundary, within the

range C0000h to E8000h, default is D0000h.

2)   FEATURES OF THIS RELEASE

========================

The CreditCard Token Ring Adapter IIps has the following new features:

- Windows- and DOS-based installation program automatically configures

PC and NOS setup

- Windows-based XView utility program provides instant status about

PC resources (Card Services information), Xircom resources

(configuration program information), CreditCard Adapter information,

and diagnosis of conflicting information to help assist

troubleshooting

- Improved driver performance

3)   MICROSOFT NOTES

===============

Ndis3 drivers are for Windows for Workgroups and Windows NT are available

on this release.

4)   OS/2 NOTES

==========

The OS/2 configuration utility, CTCFG.OS2, was not available

at release time.  It will be available on the Xircom BBS when

it is released.

Without CTCFG.OS2, the CTND.OS2 and CTODI.SYS drivers for OS/2

will NOT work properly with Socket Services or Card Services

for OS/2.  Until CTCFG.OS2 is available, Socket Services and

Card Services should be commented out of the CONFIG.SYS as follows:

rem DEVICE=C:\path\SIBMxxx1.SYS

or

rem DEVICE=C:\path\SIBMxxx2.SYS

or

rem DEVICE=C:\path\SSxxxxxx.SYS

rem DEVICE=C:\OS2\PCMCIA.SYS

rem DEVICE=C:\OS2\MDOS\VPCMCIA.SYS

CTND.OS2

--------

If CTND.OS2 loads properly but an error similar to the following occurs:

PRO0021: A general failure occurred when the program tried to

bind NETBEUI to XIRCT_NIF

ensure that the CreditCard Adapter is properly installed, configured,

and physically connected to the network, then reboot your machine.

If the problem persists, contact Xircom Technical Support.

5)   AS/400 NOTES

============

Problems using PC Support Version 2 Release 3 have been fixed.

6)   CTTEST.EXE & CTSETUP.EXE CONFIGURATION

======================================

CTTEST.EXE & CTSETUP.EXE can be configured for host hardware settings

different from the defaults using CTCFG.EXE, i.e.:

CTCFG driver=ipx port=320 irq=10

CTTEST

or

CTCFG driver=ipx port=320 irq=10

CTSETUP 4

7)   TROUBLESHOOTING

===============

If your CreditCard Token Ring Adapter is not working properly, check

the following items:

1)  'Mode' setting

2)  'Slot' setting

3)  Removing Card Services and Socket Services

4)  Using CTLSP.EXE with the 'NONETB' option

5)  Using CTLSP.EXE with IPXLS.COM

'Mode' setting

--------------

The 'mode' parameter is used to specify either I/O mode or memory-

mapped mode of data movement.  Some machines only work in I/O mode.       To configure a machine for I/O mode, the keyword 'mode 1' must be used in

the NET.CFG file for CTODI.COM, or 'mode = 0x01' in the PROTOCOL.INI

file for CTND.DOS.  If you are using CTIPX or CTLSP, 'mode=0x01' must

be specified on the command line for CTCFG, and 'mode=1' must be

specified on the command line for CTIPX or CTLSP.

See the User's Guide for more information regarding the 'mode'

parameter and example configurations.  See the section below titled

'Machine Specific Notes' for a list of machines which require

specification of I/O mode.

'Slot' setting

--------------

Many machines have more than one PCMCIA slot.  The CTCFG utility and

drivers default to slot 1.  If you wish to use a different slot, the

CreditCard software must be configured to specify the appropriate

slot.

When using the ODI drivers (CTODI.COM or CTODI.SYS), edit your        NET.CFG

to specify the slot in which the CreditCard adapter is installed by

adding the line "socket 1" for slot 1 and "socket 2" for slot 2.

When using the NDIS drivers (CTND.DOS or CTND.OS2), edit your

PROTOCOL.INI to specify the slot in which the adapter is installed

by adding the line "slot = 1" for slot 1 or "slot = 2" for slot 2.

When using CTIPX or CTLSP, specify "slot=1" or "slot=2" on the

command line of CTCFG or CTIPX\CTLSP if not using CTCFG.

See the User's Guide for more information regarding the 'slot'

parameter.

Removing Card Services and Socket Services

------------------------------------------

Some machines are being shipped with versions of Card Services and

Socket Services which do not fully comply with the PCMCIA standards.

We recommend using the CreditCard drivers without these Card and

Socket Services.  On these machines, edit your CONFIG.SYS or

AUTOEXEC.BAT files to remark out the lines that load Card and Socket

Services.

Some machines have early versions of Socket Services resident in the

system BIOS.  If your machine is configured in this manner and your

CreditCard Token Ring Adapter is not working properly, you will

need to add the keyword 'socketservices NO' to your NET.CFG file

(for ODI drivers), or 'socketservices = NO' to your PROTOCOL.INI

file (for NDIS drivers).  For the LAN Support Program (CTLSP.EXE)

or the IPX driver (CTIPX.EXE), load the drivers as follows:

ctcfg driver=lsp socketservices=n

ctlsp

or

ctcfg driver=ipx socketservices=n

ctipx

Information on your machine's Card and Socket Services should be

included with the machine's documentation.  For further assistance,

contact Xircom Technical Support.

Using CTLSP.EXE with the 'NONETB' option

----------------------------------------

If you are using CTLSP.EXE with the NONETB option and experiencing

problems, try loading CTLSP.EXE without the NONETB option.  If the

problems persist, contact Xircom Technical Support.

Using CTLSP.EXE with IPXLS.COM

------------------------------

If you are using CTLSP.EXE with IPXLS.COM and experiencing problems,

there are alternative methods of configuring your system for both

NetWare and host communications.  For more information on alternative

configurations, see the 'Novell NetWare' section in the User's Guide.

8)   MACHINE SPECIFIC NOTES

======================

*** If Card and Socket Services are not installed and you are   ***

*** using a memory manager, be certain to exclude the memory    ***

*** being used by the CreditCard adapter. Exclude an 8k range.  ***

***        example:   device=c:\dos\himem.sys                   ***

***                   device=c:\dos\emm386.exe x=D000-D1FF      ***

*** This example excludes the D0000-D1FFF memory block that is  ***

*** used by the CreditCard in a default configuration.          ***

Some machines may experience problems when using the CTIPX.EXE,

CTLSP.EXE, and CTND.DOS drivers in their default mode.  In this

situation, I/O mode should be used.  Set the machine into I/O

mode by following the instructions in the 'mode' setting portion

of the TROUBLESHOOTING section located earlier in this

README.DOC.  Some of the machines with this problem include:

IBM ThinkPad 720

ZEOS

Gateway 2000 Colorbook

NCR 3150

Sharp 6700 (model #6781)

Siemens Nixdorf PCD-4Nsl

Toshiba T3300SL

CompuAdd 425

------------

The Xircom CreditCard Token Ring Adapter currently does not work

with this machine.  Contact Xircom Technical Support for possible

workarounds.

Dell 320SLi

-----------

The Dell 320SLi may have an early version of Socket Services resident

in the system BIOS that is not compatible with newer PCMCIA software

specifications.  In this case, either the NET.CFG file (for ODI drivers)

must be edited to include the keyword string 'socketservices NO', or

the PROTOCOL.INI file (for NDIS drivers) must include the keyword string

'socketservices = NO'.  To use the LAN Support Program (CTLSP.EXE) or

the IPX driver, load the drivers in the following manner:

ctcfg driver=lsp socketservices=n

ctlsp

or

ctcfg driver=ipx socketservices=n

ctipx

Gateway 2000 Colorbook

----------------------

If you are using CTODI.COM and VLM.EXE with NetWare 4.x, in addition

to a memory manager such as EMM386.EXE, and are experiencing problems,

there are two alternatives:

1)  Use NETX.EXE instead of VLM.EXE

2)  Try removing the memory manager and rebooting your machine.

If neither alternative is successful, contact Xircom Technical Support.

Sharp 6700 (model #6781)

------------------------

This machine only works in I/O mode.  Set the machine into I/O mode by

following the instructions in the 'mode' setting portion of the

TROUBLESHOOTING section located earlier in this README.DOC (also

shown in the following example).

We recommend using memory segment D000 for the CreditCard Adapter

on the Sharp 6781 (the default memory assignment).

The Sharp 6781 may have an early version of Socket Services resident

in the system BIOS that is not compatible with newer PCMCIA software

specifications.  In this case, either the NET.CFG file (for ODI drivers)

must be edited to include the keyword string 'socketservices NO', or

the PROTOCOL.INI file (for NDIS drivers) must include the keyword string

'socketservices = NO'.  To use the LAN Support Program (CTLSP.EXE) or

the IPX driver, load the drivers in the following manner:

ctcfg driver=lsp socketservices=n mode=0x01

ctlsp mode=1

or

ctcfg driver=ipx socketservices=n mode=0x01

ctipx mode=1

The Sharp BIOS should also be configured.  On page 2-of-4 in the setup,

underneath the section heading "Floppy Drives & IC Card," configure

the third entry as follows:

Device 3 (Other):   IC Card COM Port:   COM3 I/O 3E8H, IRQ 5

Also on page 3-of-4 in the setup have the heading EMS Pageframe: DISABLED

Toshiba T3300SL

---------------

This machine only works in I/O mode.  Set the machine into I/O mode by

following the instructions in the 'mode' setting portion of the

TROUBLESHOOTING section located earlier in this README.DOC.

On the Toshiba T3300SL, the BIOS also must be set up to accommodate

I/O cards.  To set up the PCMCIA slot, select the SETUP option in the

program TEST3.EXE.  Change the option on the 'Card Modem' entry to

'Not Used'.  This prevents BIOS from enabling the PCMCIA slot for a

proprietary Toshiba modem card.

Toshiba (other models)

----------------------

Some Toshiba machines have earlier versions of Card and Socket Services

that are not compatible with newer PCMCIA software specifications.  We

recommend using these machines without their Card and Socket Services.

Refer to the TROUBLESHOOTING section earlier in this README.DOC for

information on how to use the adapter without Card and Socket Services.

9)   XIRCOM TECHNICAL SUPPORT

========================

To obtain technical support for your Xircom product, please call or send

a facsimile to the appropriate number listed below.  Access the Xircom

INFOSYS BBS at up to 14400 bps, No parity, 8 data bits, 1 stop bit

(N-8-1).  Use ANSI or TTY for your terminal emulation (the Xircom BBS

PCs do not support DEC VT-type terminal emulations).  During the Beta

process, please direct all comments and questions to Dave Green, the

Beta Coordinator at 818-878-6492 or fax to 818-878-7175.

Corporate Offices        Beta Support and Information

------------------------ ---------------------------------------------

Xircom, Inc.             United States, Dave Green  TEL: 818-878-6492

Corporate Headquarters   International............. TEL: 818-878-6280

26025 Mureau Road        24-Hr Fax-on-demand.(USA). TEL: 800-775-0400

Calabasas, CA  91302     24-Hr Fax-on-demand (Intl) TEL: 818-878-6400

TEL: (818) 878-7600   24-Hr Fax................. FAX: 818-878-7175

FAX: (818) 878-7630   24-Hr BBS Data Lines...... BBS: 818-878-7618

24-Hr Internet E-mail..... INT: cs@xircom.com

24-Hr CompuServe.......... CIS: GO XIRCOM

HOURS:  6:00 a.m. to 4:30 p.m., Monday through Friday (Pacific)

Xircom Europe NV         Europe, Mid-East, Africa.. TEL: 32(0)3 360 38 60

Bisschoppenhoflaan 82-84 24-Hr Fax................. FAX: 32(0)3 326 31 50

2100 Antwerp, Belgium    24-Hr BBS Data Lines...... BBS: 32(0)3 326 23 68

TEL: 32(0)3 326 34 94

FAX: 32(0)3 326 31 50

HOURS:  8:30 a.m. to 5:30 p.m., Monday through Thursday

8:30 a.m. to 4:30 p.m., Friday

Xircom Asia Ltd.         Asia Pacific.............. TEL: 852-525-2078

1604 Century Square      24-Hr Fax................. FAX: 852-525-8955

1-13 D'Aguilar Street    24-Hr Data Lines.......... BBS: 852-537-6048

Hong Kong

TEL: 852-525-2078

FAX: 852-525-8955

HOURS: 9:00 a.m. to 5:00 p.m., Monday through Friday

-----------------------------------------------------------------------

Thank you for making Xircom a part of your network.

-----------------------------------------------------------------------

Xircom acknowledges all tradenames and trademarks used in this document

as the property of their respective owners.

===============================[  END  ]================================

#