#*============================================================================

#*      MODICON, Inc., 1 High Street, North Andover, MA 01845, USA

#*      Copyright (c) $Date: 1996/03/29 13:35:37 $ - All rights reserved.

#*

#*      No part of this document may be reproduced in any form

#*      without the express written consent of MODICON, Inc.

#*

#*=fdoc=======================================================================

#*

#* File:        readme.doc

#*

#* RCS:

#* $Revision: 1.1 $

#* $State: Exp $

#* $Locker: $

#*

#*=fdoc=====================================================================*/

===============================================================================

Part Number: 043508181, rev. 1.30

MB+ DOS Device Driver version: 3.43

ECO #18179, Aug. 1996

Release Notes:

1) Driver Code still at rev. 3.43.

2) Only product changes are:

a) The product is now distributed on one 3.5" (1.44mb)

diskette instead of two double density 3.5" diskettes and

two double density 5.25" diskettes

b) Corresponding changes to read.me and packing.lst files

c) Testsrc directory contains utility files previously

corrupted on disk 2 of 2 in product rev. 1.20 distribution

d) The sample program test4.c has a corrected timeout value

of 10 in ncb_receive_wait (was erroneously set to 0)

===============================================================================

1 INTRODUCTION

This diskette contains DOS device drivers and associated files required

to access the Modbus Plus network via Modicon IBM Host Based Devices.

The OS2 Modbus Plus driver is distributed on a separate disk and is not part

of this product.

Note:  All IBM Host Based Devices use MBPHOST.SYS.

For information about the files contained on this diskette, please

refer to the enclosed file PACKING.LST.

2 NOTES ON THE MODBUS PLUS DEVICE DRIVERS

2.1 RELEASE NOTES

Version 3.43 contains the following fixes or enhancements:

1. Fixed a bug in interfacing to AT-984-2 and AT-984-4 Modicon

controllers.

2. Lengthened the timeout for get_response_wait.

3. Added protection for wrong NCB_NUM's.

4. Added more error checking.

Version 3.42 was an internal release.

Version 3.41 contains the following fixes or enhancements:

1. Support for the MB+ PCMCIA controller has been added to MBPHOST.SYS.

This is implemented via the /p and /a switches in the CONFIG.SYS

entry. See the PCMCIA documentation for details.

2. A bug in version 3.40 of MBPHOST.SYS in reading global data has been

fixed in 3.41.

3. MBPLUS.DLL now returns ERR_sess_table_full if GlobalDOSAlloc fails

when a buffer to send to the driver is being allocated. (Rev 4.11)

Version 3.40 contains the following fixes or enhancements:

1.	All test source and library source has been ported to use Borland's

3.1 C compiler. Makefiles with the extension .mak have been provided

on the installation disk. As in previous releases, MS 5.1

compatible makefiles are also provided.

2. Integration of the MBPLUS DLL. The DLL code has been merged with

the OS/2 and DOS versions of netlib. The source code, makefiles, and

include files have been included in the netlib and mbphinc

sub-directories of the installation disk.

To have the SA85 work under Windows for Workgroups it is necessary to

change the software interrupt used by MBPHOST.SYS and MBPLUS.DLL to

something other	than 5C (the default).  To change the interrupt it is

necessary to change the settings for both the device driver (by

using the /S switch in config.sys) and for the MBPLUS.DLL (by adding

a PORTS section to the modicon.ini file). The entry in the [PORTS]

section is of the form

MBPX=YY

where X is the adapter number (0 or 1) and YY is the software

interrupt in hex.

If two adapters are to be run simultaneously (MBP0 and MBP1) under

Windows they must BOTH use the same software interrupt. If there is

no MODICON.INI file, the interrupt may be changed by the application

by using the ncb_set_sw_interrupt() call.

Please note that if you change the software interrupt from the 5C

default, then it is also necessary to use the /S switch when invoking

the utility programs sa85off and bdreset.

Example to use interrupt 5D:

config.sys

----------

DEVICE=C:\SA85\MBPHOST.SYS /MCC00 /N0 /S5D /R2

modicon.ini

-----------

[PORTS]

mbp0=5d

sa85off.exe

-----------

SA85OFF.EXE /S5d

3. SA85.SYS, SM85.SYS and MBPHOST.SYS have been merged. The device

driver determines the device type at initialization time. This

enables a single driver (MBPHOST.SYS) to handle multiple board types.

4.	A significant reduction in the memory usage of the MBPHOST.SYS

driver has been achieved. The loaded driver now occupies approx.

16K of memory.

5. An effort has been made to lessen the amount of processing done

during each timer tick interrupt. This should alleviate compatibility

problems with programs which also depend on the timer.

2.2 INSTALLATION OF THE DEVICE DRIVER

To install the device driver software, place the appropriate device

driver files on your boot disk.  For all IBM Host based devices, use the

device driver file MBPHOST.SYS.  Modify your CONFIG.SYS file to contain

something like the following:

DEVICE=MBPHOST.SYS /md000 /n0 /r2

Where the switch options are as follows:

/mXXXX    Selects memory window setting.  XXXX represents the

hexadecimal paragraph address of the memory on the board.

This means that the physical address of the board memory is

at address XXXX0, or in segment:offset notation words XXXX:0.

For example, specifying /mD000 means the board will occupy

a 2 Kbyte region at address D000:0.  The default memory

window is /mD000.

XXXX must be on an 80 hex boundary in the range C000 -

FF80, and must correspond to your board address settings.

/nD       Selects the LAN adapter number to use.  D must be either 0

or 1.  The default value is 0.  If using more than one

board in a system, be sure that a device driver is loaded

for each board, and the LAN adapter number is unique for

each copy of the driver.  The LAN adapter number must be

specified in programs that communicate to the device

driver.  It is independent of memory window setting.

/sXX      Selects the software interrupt to use to communicate with

the device driver.  The default value is 5C hex.  This

switch should only be used if there is a conflict with

another device already using software interrupt 5C hex.  IF

THIS VALUE IS MODIFIED, ALL APPLICATIONS MUST BE MADE AWARE

OF THE NEW SETTING TO WORK.  See the documentation on your

application to find out how to change this interrupt.  If

you are a developer, you should provide a method for your

end users to change the software interrupt number.

/rD       Driver revision level compatibility specifier.  D must be

either 1 or 2.  The default value is 1.  This switch

redefines the implementation of the ncb_receive function.

If using /r1, the ncb_receive function will wait for a

response until a timeout occurs (identical to

ncb_receive_wait, using the timeout value from the NCB_RTO

field of the NCB).  If using /r2, the ncb_receive function

will return immediately with status regarding the

transaction in progress.

/bD       Board reset and SA85 off inhibit.  D must be either 0 or 1.

The default value is 0.  If set to 1, the ncb_reset and

ncb_sa85off functions will be disabled.  A call to either

function will have no effect.  When operating under Microsoft

Windows 3.x and Quarterdeck DESQView, these functions are

automatically disabled regardless of the state of this

switch.

2.3 MODBUS PLUS DRIVERS USE OF THE PC TIMER TICK

The Modbus Plus driver is required to do the following processing

autonomously:

a) kick the board approximately every second

b) in the absence of a host application owning the slave paths, it

must service any slave requests with routing failures, otherwise the

side issuing the slave request will hang INDEFINITELY.

To accomplish these, it is necessary for the driver to attach itself

to the PC's timer tick, so that at appropriate times it can service

these two functions.

If the use of the driver is contemplated along with other software

which also either attaches to the timer tick, or re-programs the

timer tick, then compatibility testing should be performed to ensure

that both pieces of software coexist peacefully.

Products which fall into this category include multi-taskers like

DESQview, and also products with built-in multi-tasking, like

Intellution's FIX package.  Both these products also use the timer

tick and need to be tested with the current driver. The driver

has been tested with DESQview and the Intellution FIX package, and

operates satisfactorily in those environments.

2.4 OTHER CONSIDERATIONS

We have taken advantage of the flexibility of the Netbios paradigm in

providing means for specifying the type of Modbus Plus "Path"

desired, out of the four types Program Master, Data Master, Program

Slave, and Data Slave. We have provided for this required

functionality by having the requested path type part of the

destination node name.

Most NetBios commands have two forms determined by the most

significant bit of the command byte.  In one form, the application

waits until the command is complete.  In the other form, the

application issues the command and the command returns immediately,

following which the application can periodically check the status

field in the NCB to test for completion.  This implementation only

implements the commands so they wait until completion, except for the

RECEIVE and RECEIVE_WAIT functions, which are implemented for both

styles.

==========================================================================

A P P E N D I X   A

Recommended DESQview settings:

Recommended DesqView settings for running MBPHOST.SYS Modbus plus device

driver with MBPSTAT or other applications

=======[   1    ]=========================================================

Advanced Setup: Performance

Task Processing Time (in Clock Ticks)

Foreground:    3

Background:    3

Memory Usage (in K)

Common Memory:      12

DOS Buffer for EMS:  2

Optimize communications?    (Y/N): N

Allow swapping of programs? (Y/N): Y

Manage printer contention?  (Y/N): N

=======[   2    ]=========================================================

Advanced Setup: Video Monitor

What display adaptor do you have?  2

Monochrome                       0

Color Graphics (CGA)             1

Hercules                         2

Enhanced Graphics (EGA or VGA)   3

MultiColor Graphics Array (MCGA) 4

Other                            5

Current Graphics Driver: GRFHERC.DVR

Do you want text & graphics displayed

at the same time? (Y/N):  N

Does your display adaptor require

synchronized access? (Y/N):  N

Blank the screen after  0 minutes

of inactivity--enter 0 for never.

=======[   3    ]=========================================================

Program Name............: mbpstat

Keys to Use on Open Menu: MB                         Memory Size (in K): 120

�����������������������������������������������������������������������������

Program...: c:\bin\exe\mbpstat.exe

Parameters:

Directory.: c:\bin\exe

�����������������������������������������������������������������������������

Options:

Writes directly to screen......: [N]

Displays graphics information..: [N]

Can be swapped out of memory...: [Y]

Requires floppy diskette.......: [N]

Advanced Options:

System Memory (in K).......:   0   Maximum Program Memory Size (in K)..:

Script Buffer Size.......:  1000   Maximum Expanded Memory Size (in K):

Text Pages: 1  Graphics Pages: 0   Initial Mode:        Interrupts: 00 to FF

�����������������������������������������������������������������������������

Window Position:

Maximum Height:  25       Starting Height:  12       Starting Row...:  10

Maximum Width.:  80       Starting Width.:  80       Starting Column:   0

�����������������������������������������������������������������������������

Shared Program

Pathname..:

Data......:

�����������������������������������������������������������������������������

Options:

Close on exit to DOS.........: [Y]      Uses its own colors.........: [Y]

Allow Close Window command...: [N]      Runs only in foreground.....: [N]

Uses math coprocessor........: [N]      Keyboard conflict (0-4).....: [0]

=======[   4    ]=========================================================

Program Name............: DOS (128K)

Keys to Use on Open Menu: D1                         Memory Size (in K): 128

�����������������������������������������������������������������������������

Program...: prompt

Parameters: $p$g

Directory.:

�����������������������������������������������������������������������������

Options:

Writes directly to screen......: [N]

Displays graphics information..: [N]

Can be swapped out of memory...: [Y]

Requires floppy diskette.......: [N]

�����������������������������������������������������������������������������

Advanced Options:

System Memory (in K).......:   0   Maximum Program Memory Size (in K)..:

Script Buffer Size.......:  1000   Maximum Expanded Memory Size (in K):

Text Pages: 1  Graphics Pages: 0   Initial Mode:        Interrupts: 00 to FF

�����������������������������������������������������������������������������

Window Position:

Maximum Height:  25       Starting Height:           Starting Row...:

Maximum Width.:  80       Starting Width.:           Starting Column:

�����������������������������������������������������������������������������

Shared Program

Pathname..:

Data......:

�����������������������������������������������������������������������������

Options:

Close on exit to DOS.........: [N]      Uses its own colors.........: [N]

Allow Close Window command...: [Y]      Runs only in foreground.....: [N]

Uses math coprocessor........: [N]      Keyboard conflict (0-4).....: [0]

=============================================================================