MPHOS2.DOC for OS/2 Modbus Plus Driver

043512124 version 2.10				12/11/96

Disk Assembly for OS/2 MB+ Driver, including:

MPHOST.SYS version 3.1		-- OS/2 MB+ Driver

MBBRIDGE.EXE version 1.2	-- Dos Bridge

MPHOST32.DLL			-- 32-bit DLL

SA85.DLL			-- 16-bit DLL

Device driver MPHOST.SYS and Dynalink Libraries SA85.DLL and

MPHOST32.DLL for SA85 Modbus Plus board, for use on IBM/MS OS/2

operating system.

-- Copyright 1996 Schneider Automation --

========================================================================

This document lists:

1) CONTENTS: - what files are included in the release

2) OVERVIEW of DEVICE DRIVER and INSTALLATION PROCEDURE

3) MBPSTAT Network Monitor program for OS/2 -  Description and

Instructions

4) EXAMPLE PROGRAMS

test4 - reads 125 registers from 685

test4b - uses two SA85 boards at the same time, reads 125

registers from 2 different nodes

testslav + test4c - tests "slave" functionality

- testslav runs on one PC servicing slave requests

- test4c runs on another PC make requests of testslav

globtest - generates global data

netlib.c - netlib library routines used by example programs

5) INSTRUCTIONS ON RUNNING THE EXAMPLE PROGRAMS

6) RESTRICTIONS

<<1. C O N T E N T S >>

Among other files, this disk contains :

MPHOST.SYS      	Modbus Plus OS/2 device driver, rev 3.1

SA85.DLL        	DynaLink library to support Modbus Plus communication

for 16-bit applications

MPHOST32.DLL    	DynaLink library to support Modbus Plus communication

for 32-bit applications

SA85.LIB        	SA85/OS2 exports for SA85.DLL

MBPSTAT.EXE     	Modbus Plus Network Monitor program

MBBRIDGE.EXE    	DOS TSR, rev. 1.2,  to allow DOS apps. to use OS/2

device driver

NETLIB Library Source:

NETBIOS.H    header file required by all included source

NETLIB.H     header file required by all included source

NETLIB.LIB   Library required for using dynalink library

NETLIB.C     Source code for NETLIB

NETLIB       Makefile

Example Programs:

Source   		Makefile       Executable      OS/2 Exports

BDRESET.C    	BDRESET        BDRESET.EXE

TEST4.C    		TEST4          TEST4.EXE       TEST4.DEF

TEST4B.C     	TEST4B         TEST4B.EXE      TEST4B.DEF

TEST4C.C     	TEST4C         TEST4C.EXE      TEST4C.DEF

TESTSLAV.C   	TESTSLAV       TESTSLAV.EXE    TESTSLAV.DEF

TESTSLAV.H

READNET.C    	READNET        READNET.EXE     READNET.DEF

MBPCHECK.C   	MBPCHECK       MBPCHECK.EXE    MBPCHECK.DEF

GLOBTEST.C		GLOBTEST       GLOBTEST.EXE    GLOBTEST.DEF

VIOUTIL.DLL

<<2.  OVERVIEW of DEVICE DRIVER and INSTALLATION PROCEDURE >>

NOTE:

In this document, in many places "/OS2" has been appended to the name

"SA85", to distinguish the OS/2 version from the DOS version of the SA85

software. The term SA85 is used generically to refer to the host adapter cards

that have been developed for the IBM AT and PS2.

Also, the device driver works in conjunction with the SA85.DLL or the

MPHOST32.DLL Dynalink libraries, so both components should really be viewed as

being one integral system. Both are requisite to the correct functioning of

Modbus Plus/OS2.

2.1 INSTALLATION INSTRUCTIONS - Hardware

----------------------------------------

Refer to the user's manual for instructions on hardware installation:

"Modicon IBM Host Based Devices User's Guide",  part number 890 USE 102 00.

2.2 INSTALLATION OF THE DEVICE DRIVER

-------------------------------------

The SA85/OS2 software currently can support a maximum of four SA85 boards.

Multiple SA85's may not use the same node address or memory window.

Device driver command line options:

/B		Refuse external RESET command

/Px		Board is PCMCIA type, where x equals the slot number

/Axx	If  /P is specified, this specifies network LAN number

/Mxxxxx	Selects memory window setting.

If the /M option is not specified, the device

driver defaults to a window at D0000 hex for the

first SA85 board, and D0800 hex for the second SA85

board.

Example:  /MD000  for the window at D0000 hex

Valid window settings run from C0000 through FF800

hex at 80 paragraph boundaries.

For example:

C0000

C0800

C1000

C1800

.....

FF000

FF800

NOTE:

On most AT's the range occupied starting at E0000 is taken by

PROM space, and hence is unuseable for the SA85.  However,

some PC's, including the C996 Co-Pro have this range open, and

so it is useable for the SA85.

If the "/M" directive is not included, when the system is booted, a

message indicates the default value used and displays an example of

correct syntax.

[IMPORTANT! For version 2.1, the /i option has been disabled.]

/i  Select interrupt level.  Valid numbers are 3 to 7.

Be sure to select an interrupt level that is not used by any other

devices. If the "/i" directive is not included, poll mode is

assumed.

To install the device driver software:

1) Place the MPHOST.SYS file from the distribution floppy into your

system file directory on the PC.  Then add lines similar to the

following to the CONFIG.SYS file:

SET MPHOST0=/MD0000

SET MPHOST1=/MD0800

SET MPHOST2=/MD1000

SET MPHOST3=/MD1800

DEVICE=mphost.sys

where:  D0000, D08000, D10000 and D18000 are the memory

windows addresses for boards 0,1,2, and 3

and	MPHOST0 thru MPHOST3 refer to the four possible MB+

adapter boards.

Example 1:

To install the driver for PCMCIA, add lines similar to the following

to the CONFIG.SYS file:

SET MPHOST0=\MD0000 /P /A12

DEVICE=mphost.sys

where /P indicates PCMCIA adaptor type

and /A12 indicates LAN number 12

Example 2:

To install the driver to support two SA85 boards in your PC, add

lines similar to the following to the CONFIG.SYS file:

SET MPHOST0=\MD0000 /B

SET MPHOST1=\MD0800

DEVICE=mphost.sys

where /B indicates the external reset commands will be

refused for board 0.

The following other parameters in your OS/2 config.sys file are also

important, and should be set similar to the following:

threads=512

memman=SWAP,MOVE

maxwait=3

timeslice=32,100

2) Next, copy SA85.DLL and MPHOST32.DLL to one of the directories

specified by your "libpath" directive in your os/2 config.sys file.

"libpath" determines where OS/2 will search for .DLL files.  The root

directory and the \os2 directory are commonly used.  "libpath" is NOT

an environment setting (it cannot be set with the "SET" directive).

It must be set from the config.sys file.

2.3  INSTALLATION of DOS BRIDGE

See file READ.ME.

2.4  OTHER COMMENTS:

--------------------

SA85

----

The SA85 is an IBM-PC expansion board which interfaces to the Modbus Plus

LAN.  It has an on-board processor and an area of shared RAM, and

supervises the communication of Modbus and other messages between an

application running on an IBM-PC or compatible, and other nodes on the

Modbus Plus network.

The SA85/OS2 Netbios Device Drivers and Dynalink libraries provide a means

by which application programs running on the IBM-PC can communicate with

the firmware on the SA85 in an easy and efficient manner.

The device drivers and Dynalink libraries relieve the applications

programmer of the necessity of attending to the many details required to

properly interface to, and service, the firmware on the SA85 board.

The function of any device driver is to insulate application programs

running on the computer from dealing closely with the details of the

hardware device, thus promoting ease-of-use and portability. Should the

details of the SA85 board itself change, all that is needed is for a new

version of the driver to be written, compatible with the old standard,

resulting in minimization of the effort required to get old applications

running with a new board or operating environment.

The Netbios compatible interface for OS/2 is identical to that of DOS.

Refer to the Modicon IBM Host Based Devices User's Guide for detailed

information.

Features of the SA85/OS2 NetBIOS device driver:

o	Installs easily, via the normal method of  adding an entry to the

config.sys file. Options in the config.sys entry permit choice of

memory window address to be used

o	The SA85/OS2 device driver, in conjunction  with the SA85.DLL Dynalink

library and the MPHOST32.DLL Dynalink library, perform the required

watch-dog function of "kicking" the board approximately every two

seconds, relieving the applications programmer of this time-dependent

chore.  This design is meant to ensure minimum adverse impact on scan

time should an SA85 node become damaged  in a non-passive state. In

this event there would be a one-time only hit on the network of

approximately one second, following which the damaged node would

become a passive node, and normal network traffic would continue.

o	The SA85/OS2 device driver performs the required  servicing of slave

paths in the absence of an application program claiming ownership of

such paths.

o	Once the driver is loaded, the driver is ready to receive commands (to

be described below) from application programs.

o	The driver validates requested operations for legality before

submitting them to the firmware and reports back appropriate error

status should the requested operation not be correct.  Should the

firmware interface crash for any reason, the device driver will

detect this and notify the user with information identifying the cause

of the event.  This should not normally happen.

Crash-code:     type        description

0x1             interface   2.5-second interface timeout

0x2             interface   bad interface opcode

0x3             interface   interface data error

0x4             interface   interface test error

0x5             interface   interface transfer-done error

0x6             interface   bad interface path

0x7             interface   bad transfer state

0x8             interface   bad transfer length

0x9             interface   global-data length error

0xA             interface   global-data address error

0xB             interface   global-data not present

0x81 - 0x97     internal    internal fatal error

o	Because of the use of the SA85.DLL and the MPHOST32.DLL Dynalink

libraries, memory requirements are greatly reduced.

All tasks running and calling on the SA85 Modbus Plus/OS2 services will

share the same code segments (and some data segments) as the other

tasks.

In effect, only one image of the SA85/OS2 support software is used,

even if 10 tasks are running.

In addition, if multiple copies of user tasks are running, for example

5 copies of the MBPSTAT network monitor program, only 1 copy of the

code segments is shared by all 5 instances of the program.

This is an example of OS/2's superior memory management compared to

DOS.  In addition, if the need arises, unused segments are

automatically swapped to disk, providing true virtual memory operation.

------ Using the SA85.DLL: -----------

Please examine the enclosed TEST4 program as an example

of a program that calls upon the services of SA85.DLL.

TEST4.C calls routines which are included in NETLIB.C,

also, included.  All the routines in netlib.c call the

function "exec_netbios()", as they did in the DOS

version.  However, instead of exec_netbios() executing a

software interrupt, as was done under DOS,

exec_netbios() simply calls the function

"sw_int_handler()", which is in fact located in the

SA85.DLL Dynalink library.

When TEST4 is linked, it tells the linker, via entries

in the SA85.LIB file, that it wishes to import the

routine "sw_int_handler", and that the name of the .DLL

library containing that entry point is SA85.DLL.

When TEST4.EXE is run, OS/2 searches the directories

specified in the "libpath" directive in the OS/2

config.sys file for SA85.DLL.  If it does NOT find it, an

error message is issued.  If it DOES find it, the

reference to sw_int_handler is resolved, and TEST4

executes.

At the time SA85.DLL is attached to, OS/2 additionally

checks to see if some other task has already loaded the

required segments into memory, and if so, OS/2 simply

updates its tables so that the new copy of TEST4 uses the

same code segments already in memory.

<<3.  MBPSTAT Network Monitor program for OS/2 >>

This document describes all aspects of the "MBPSTAT.EXE"

program for use with MODBUS PLUS Networks.

INTRODUCTION

The MBPSTAT program is a MODBUS PLUS NETWORK test program

designed to collect and display various bits of information

about the MODBUS PLUS Network it is connected to.

It is essentially identical to the DOS version of MBPSTAT.

This program allows the user to do such things as display

node maps, set routing paths, and monitor error counters.

The program is menu driven, and very easy to use.  This

program communicates to the network via the NETBIOS device

driver (mphost.sys and SA85.DLL dynalink library), which must

be loaded for the program to operate.

REQUIREMENTS

This program runs in the OS/2 environment, on an 80286,

386 or 486-based computer, with a MODICON MODBUS Plus

adaptor card (both AT-bus and Micro-channel bus version).

PROGRAM INSTALLATION

In order for the MBPSTAT program to operate, the "mphost.sys"  device

driver must be present.  In addition, the SA85.DLL dynalink library (for

16-bit applications) or the MPHOST32.DLL dynalink library (for 32-bit

applications) must be installed, as previously described in the

Installation Section.

[NOTE: For release 2.1, the MBPSTAT.EXE distributed is a 16-bit

application.]

In order to invoke the MBPSTAT program, type the

following command :

C>MBPSTAT

(The command line options available with the DOS version

of MBPSTAT are not applicable or used in the OS/2

environment.)

PROGRAM OPERATION

When the MBPSTAT program is loaded, the first thing is

does is prompt the user for the "MODBUS PLUS ADAPTER"

number, and "ROUTING" information.

The "MODBUS PLUS ADAPTER" number is a number between 0

and 255 decimal, and must correspond to the adapter

number specified in the CONFIG.SYS file.  The default

value is zero.

(Pressing ENTER will cause the default value of 0 to be

used)

The ROUTING information is used for addressing remote

networks.  Up to four routing bytes may be entered.  The

first zero byte entered terminates the routing field.

(Pressing ENTER at the first routing prompt will cause

the default value of "the local network" to be used)

So for the first SA85 board, and the local network,

simply press ENTER twice when MBPSTAT starts up, to get

to the main menu.

The routing address will remain the same until the user

modifies the routing information by choosing the

appropriate menu selection.

Once the ADAPTER number and ROUTING information have

been entered, the program will display a menu of choices

of things to do.  In order to select an option, either

type the first letter of the first column of the

selection, or use the cursor keys to move to a

selection, and then press ENTER.

The "ESCAPE" key is used to stop any sub-program, and

return to the previous menu.

Once a selection has been made, the user will usually be

prompted for a node number.  This information is needed

to complete the ROUTING path.  If the ROUTING path was

set to the following:

13.12.11.00.00

The node number chosen would be added to the routing

path to create the actual node address.  If node 10 were

selected, the routing address would be:

13.12.11.10.00

The first three bytes in this case are the network

address, and the last byte is the node within the

network address.

During each test, the upper right corner of the display

will show the number of times the test was successful,

and the number of times the test failed.  If an invalid

routing address was entered, and the program was unable

to communicated with the specified node, an error

message:

Unable to read Node 13

will appear for a short time.  The program will wait for

approx. 1 second, and then try again.

For the purposes of documentation, the term "target

node" will refer to the node specified by the ROUTING

path, and the term "local node" will refer to the node

address of the board installed in the computer running

the MBPSTAT program.

SET ROUTING PARAMETERS

This selection allows the user to specify a new set of

routing parameters without exiting the program.

MONITOR NETWORK ACTIVITY

This selection is used to create a list of which nodes

exist on the current network.  This is done by

attempting to communicate with every possible node (1 to

64), and noting whether or not a response was received.

After the "nodes present" has been determined, this

sub-program will read the global data from each node,

and display the number of words of global data read.

This operation is done by reading global data at the

local node.

READ GLOBAL DATA

This selection will continuously read and display the

global data from the specified node.  If global data was

present, but was zero words in size, the "null data"

message will be seen.  If there was no global data from

the specified node, an error message will be displayed.

This operation only involves the local node.

GLOBAL DATA PRESENT

This selection displays a map of the nodes which have

global data present from the target node point of view.

A node number will appear on this map if the target node

ever received global data from the node number, and did

not read the global data up from the peer processor.

The information displayed here is not necessarily a list

of active stations.

NODE ACTIVE STATION

This selection continously queries the target node for

an active station table list, and displays the results.

The display shows a picture what other nodes exist from

the target nodes point of view.  If there is any reverse

video blocks displayed without numbers in them, this

means the node was present when the test began, but is

not currently present.

NODE ERROR STATISTICS

Each node maintains several error counters, which will

get incremented each time the specific type of error

occurs. This selection continously queries the target

node for this list of error statistics, and displays

them.  The error counters can be cleared by striking the

SPACE BAR.

TOKEN STATION TABLE

The TOKEN STATION TABLE selection displays a map of

nodes which are passing the token from the target nodes

point of view.  Token passing nodes are classified as

peers.

TOKEN OWNER WORK TABLE

The TOKEN OWNER WORK TABLE selection will display a map

of work which needs to be done by the target node the

next time it gets the token.  This work consists of

servicing the internal paths of the target node.  A

reverse video block without a number in it indicates an

internal path which has been active at least once since

the routine began, but is not currently active.  This

"one shot" effect gives the user a better indication of

the actual node activity.

CURRENT INTERNAL PATH TRANSACTION

This selection displays the actual number of

transactions processed by each of the internal paths of

the target node. Totals for each path are calculated by

adding the change in the actual count each time an

update is performed.  Total inbound and outbound

transactions are the sum of the changes in all of the

inbound or outbound transactions.  Totals may be cleared

by striking the SPACE BAR.

NODE PERSONALITY

This selection will display information about the target

node.  This information will include things like node

type, software revision level, and token rotation time.

<<4. EXAMPLE PROGRAMS >>

test4 - reads 125 registers from 685

test4b - uses two SA85 boards at the same time,

reads 125 registers from 2 different nodes

testslav + test4c - tests "slave" functionality

testslav runs on one PC servicing slave requests

test4c runs on another PC make requests of testslav

globtest - generates global data

netlib.c - netlib library routines used by example programs

COMPILING AND LINKING INSTRUCTIONS:

1) build netlib.obj and netlib.lib first using the supplied 'netlib'

makefile.

2) compile and link the example programs using the supplied

makefiles.

o     Example Source of a program which

uses the SA85/OS2 device driver:

The program below, test4.c, requests holding registers

from a 984-685 on the network. Test4.c makes use of some

simple library routines, found in netlib.c, which is

also listed here. All the testprograms were developed

using the Microsoft 5.1 "C" compiler, /Alfu memory model

(Large).  MAKE files are included for the sample

programs showing suggested compiler settings to use.

<<5. INSTRUCTIONS ON RUNNING THE EXAMPLE PROGRAMS >>

<< INITIALIZATION >>

After installing the SA85 hardware and SA85/OS2 device

driver software, as previously described, activate the

the SA85/OS2 device driver by booting the PC.

Diagnostic on the board is performed when the driver

is installed.

If the SA85 fails its internal diagnostics, the most

likely cause is that the settings for the memory

window address are not correct.  Verify that they are

correct and do not co-incide with other PC functions. If

the SA85 fails, it will display messages similar to:

"load pattern failure at ram offset %x"

"read %x, expected %x"

"response pattern failure at ram offset %x"

"read %x, expected %x"

<< RUNNING THE Example PROGRAMS >>

Five examples programs will be described:

1) mbpstat - network monitor program

2) test4 - reads 125 registers from 685

3) testslav + test4c - tests "slave" functionality

4) test4b - uses two SA85 boards at the same time

5) globtest - generate global data

Initial Setup Checklist:

1) SA85 board and driver installed in a PC AT, or 386.

2) SA85 cabled to Modbus Plus network with at least one 685.

You must know the node address of the 685.

1) MBPSTAT - network monitor program

Refer to the previous section on MBPSTAT for a full

description of the use of this program. Start the

MBPSTAT program and verify that the network can be

monitored successfully. Upon starting MBPSTAT, select

"0" for the LAN adapter number and "0" for initial

routing byte. (You can simply press ENTER twice to set

these default values.) Menu choice "6" will display

error statistics for selected node addresses. If you

have a router on the network, you may "go through the

router" with MBPSTAT to observe operating conditions on

the other side of the router.  To do this, specify the

routers address when starting MBPSTAT.

2) TEST4 - Test4 attempts to read 125 holding registers

from node x.  Ensuring you have a node "x" on your

network, start test4 by typing 'test4 x y'.  'x' identifies the

slave node receiving the request and y identifies the adaptor number.

It should be either 0 or 1.  For each request and receipt of

125 holding registers, the program will display a

running count, as well as the calculated number of

transactions per second. The program should continue

running continuously.  Any error conditions will result

in the programs terminating with an error message.  To

stop the program simply press 'Q'.

3) TESTSLAV + TEST4C

In this test, the TESTSLAV program acts as a Modbus slave,

and waits for modbus messages. It will respond to requests

to read and write single and multiple holding registers.

TEST4C is similar to TEST4 in that it will be making

requests to read holding registers from TESTSLAV.

Start TESTSLAV by typing 'TESTSLAV x y'  where x identifies

the slave path (valid range 1 - 8) to use, y identifies the

adaptor number (0 or 1).

Then run TEST4C from another Modbus Plus adaptor.  Type

'TEST4C x y z'  where x identifies the slave node address,

y identifies the slave path number in the slave node and z is

the adaptor number.

If TESTSLAV is started while TEST4C is NOT running, it

will pause, and every 15 seconds or so will display a

message "status = 0x204".  This means no slave requests

were received for this node during the preceding period.

When TEST4C is started on the other PC, the TESTSLAV

program should run much faster, displaying a running

count of slave requests serviced for each modbus message

it services.  The TEST4C program will display a "+-" for

each message sent and receipt of message back from

TESTSLAV.

If TEST4C is run without TESTSLAV running, it will

receive back a "routing failure" message and display a

message "status = 18", which indicates the routing

failure.  This is for the case where the slave node has the

SA85B0.SYS driver installed and it is on the network, but

the TESTSLAV program is not running.  For the case where

the slave node does NOT have the SA85B0.SYS driver loaded, TEST4C

will report "status = 5", which indicates a timeout

status.

4) TEST4B

This program exercises two boards in one PC at the same

time.  You MUST have 2 SA85 boards installed in one PC.

Ensure both are set to different memory window addresses

and node addresses.  For example, have them set to

/md000 and /md080 in the config.sys entries, see the

installation instructions above.  You must include a

separate copy of the driver for each board.  Example:

(in config.sys) device=mphost.sys /md000 /n0

device=mphost.sys /md080 /n1

After rebooting the PC to install the drivers, you will

have two drivers installed as LAN adapter #0 and #1.

Specify the two node addresses to talk to on the command

line:

For example,

TEST4B 2 3

will send requests to read 125 holding registers to node

2 through the first board (#0), and to node 3 through

the second (#1) board.

Each message sent and receipt of message received will

display a running count.

Any error conditions encountered will result in the

program stopping with appropriate error messages.

While you have two SA85 boards installed in one PC, you

may wish to run the MBPSTAT network monitor program,

verifying that you can select which LAN adapter board

you wish to use from MBPSTAT.  The LAN adapter number is

selected for MBPSTAT when the program is started.

MBPSTAT always displays the LAN adapter number in use

while it is running.

5) GLOBTEST

Run this program on one PC while using the MBPSTAT

program on another PC to observe the global data

generated by the node running GLOBTEST.  Under OS/2, it

is possible to run GLOBTEST, and at the same time run

MBPSTAT as another task.

<<6.  RESTRICTIONS >>

o  The SA85/OS2 driver software implements a subset

of the NetBIOS standard.  It will probably not

support applications that use all the NetBIOS

commands, such as file server or electronic mail

applications.

If a "Gateway" application is needed, write an

application program using the driver to provide

the Gateway facilities into the Novell or other

network.

o   The SA85/OS2 driver's use of the PC timer tick:

The SA85/OS2 driver is required to do the following

processing autonomously:

a) kick the board approximately every second

b) in the absence of a host application owning

the slave paths, it must service any slave

requests with routing failures, otherwise the

side issuing the slave request will hang

INDEFINITELY.

To accomplish these, it is necessary for the

driver to attach itself to the PC's timer tick,

so that at appropriate times it can service

these two functions. This function under OS/2 is

implemented via the Modbus Plus driver, as of version

2.0.

In addition, other software should not leave interrupts

off for extended periods or adjust task priorities

indiscriminantly.

o   The NetBIOS RESET command normally returns standard

field information in the result buffer.  In the current

implementation, the buffer contents do NOT follow the

standard field layout.  The RESET command is a good

command to use during error processing, as it 1) runs

diagnostics on the board, 2) returns the board's internal

firmware to a know "initial" state. Be aware that the

RESET command can take up to 100 msec. to execute, so

it should not be used indiscriminately.  During normal

processing there is no need to use the command at all.

Also, it is not adviseable to use the RESET

command when other tasks are currently running

using the SA85/OS2 services.

#