Windows 3.0 and 3.1 Driver Documentation

Aox Inc.

8/3/90

Last updated:

5/1/92

1. Purpose of Driver

MMVDMAD is a driver for Windows 3.0 and 3.1

and is required for proper operation in certain environments

when Windows is being run on an Aox MicroMASTER board.

Specifically, systems that contain a SCSI controller or

any other MCA bus master device that performs DMA will need to use

this driver for Windows to run in extended mode on our board.

If the driver is not used, Windows will fail to load fully and

eventually terminate with a disk error.

At this time, the only bus master known to perform DMA is

the IBM SCSI controller.  This controller is standard on the IBM PS/2

Models 65 and some Model 80's.

2. Installation instructions:

File MMVDMAD.386 is the actual driver for Windows 3.0 or 3.1.

The appropriate file must be copied into the SYSTEM directory

under WINDOWS and is located on the Aox option diskette.

The text file SYSTEM.INI is in the WINDOWS directory.  This file

contains entries for all drivers that Windows will load.  The following

entry is standard in this file:

device = *VDMAD

This entry should be replaced with the following line:

device = mmvdmad.386  (for Windows 3.0 or 3.1)

There is no need to reboot the system before running Windows again.

It will automatically use the new SYSTEM.INI file to load the new driver.